SMART AC RECOMMENDATION SYSTEM

INTRODUCTION :

Imagine having an intelligent assistant that not only keeps you comfortable but also helps you save energy. Our AIoT project brings this vision to life by creating a seamless connection between the outside world, your home, and smart decision-making.

We gather real-time weather data like temperature and humidity from the world outside, and at the same time, we keep track of the indoor climate using specialized sensors. We're even attentive to whether your AC is running or not, because every little detail matters.

All of this valuable information gets stored in a digital vault called a time series database. It's like a giant digital journal that captures how things change over time. But the real magic happens when we use advanced AI techniques to analyze all this data. It's like teaching our system to understand patterns and predict the future, but for home comfort!

When you have questions or need advice, our friendly AI assistant, powered by ChatGPT, is here to help. Wondering when to turn on the AC to avoid heat or humidity? Just ask! Our AI assistant dives into the treasure trove of data, looks at how the weather has been behaving, how comfy your home is right now, and even what your AC has been up to. Then, it cleverly suggests the best times to switch on the AC for maximum comfort and energy savings.But that's not all – we make sure you receive the advice in a way that suits you. Our AI can talk! You'll get to hear its recommendations spoken aloud, just like having a conversation. And if you prefer, you'll also receive the advice through your Telegram app.

What's truly exciting is that this system learns and evolves. The more people use it, the smarter it gets at predicting the best AC times. So, it's not just about making you feel cozy – it's about making homes smarter, saving energy, and contributing to a happier planet. Welcome to the future of intelligent home comfort!

HARDWARE USED:

- WIZnet-W5300 TOE SHIELD + STM32-F429ZI board

- DHT11

- Jumperwires

- Bread Board

- Microphone

- Resistor

SOFTWARE  USED:

- MQTT SERVICE

- PYCHARM

- ARDUINO IDE

PROGRAMMING LANGUAGES:

- C++

- PYTHON

APPS AND ONLINE SERVICES USED:

- MQTT BROKER

- OPENAI

- INFLUXDB

- TELEGRAM API

WORKFLOW:

1. User Voice Command:

- The user interacts with the system by giving a voice command through a device, such as a smartphone or a smart speaker. For instance, they might say, "When should I turn on the AC today?"

2. Speech-to-Text Conversion:

- The voice command is captured and converted into text using a speech-to-text technology. This transforms the spoken words into a format that can be processed by the AI system.

3. Query Sent to Data Retrieval Module:

- The converted text query is passed to a data retrieval module, which contains Python code and queries specifically designed to fetch historical time series data from the InfluxDB database. This data includes details about external weather conditions, indoor climate measurements, AC status, and other relevant variables recorded over time.

4. Data Sent to OpenAI Model:

- Once the relevant data is fetched from the InfluxDB database, it's sent to the OpenAI model as input. The data provides context for the user's query and allows the AI model to understand and analyze the user's question accurately.

5. Data Analysis by OpenAI:

- Using the received historical time series data, the OpenAI model processes the information to identify patterns, correlations, and trends. It considers factors like past AC usage, weather trends, and comfort levels to formulate a suitable response.

6. Text-to-Speech Conversion:

- After analyzing the data, the OpenAI model generates a response text suggesting the optimal time to turn on the AC. This response text is converted into speech using a text-to-speech synthesis tool.

7. Voice Response to User:

- The synthesized voice response is played back to the user through a speaker or audio output device. The user hears the AI's recommendation spoken aloud, providing a natural and convenient interaction.

8. Telegram Bot Notification:

- Simultaneously, the response generated by the OpenAI model is sent to the Telegram bot. The bot then forwards the recommendation as a text message to the user's Telegram account.

Flowchart:

SETTING UP USING PYTHON:

Setting up with MQTT:

import paho.mqtt.client as mqtt

paho.mqtt.client: This library provides a client implementation for MQTT (Message Queuing Telemetry Transport), a lightweight messaging protocol widely used in the Internet of Things (IoT) domain for communication between devices. It allows you to connect to an MQTT broker and publish messages to topics or subscribe to topics to receive messages. It's commonly used for real-time data transmission in IoT applications.

Installation: To install the paho-mqtt library, you can use pip, the Python package manager. Open your terminal or command prompt and run the following command:

pip install paho-mqtt

Setting up with OPENAI:

import openai

openai: This library provides a Python interface for the OpenAI API, allowing you to interact with various natural language processing models and services provided by OpenAI. With this library, you can access powerful language models like GPT-3 to perform tasks such as text generation, language translation, question-answering, and more.

Installation: To install the openai library, you can use pip as well. Run the following command in your terminal or command prompt:

pip install openai

Setting Up with Recogniser:

import speech_recognition as sr

This library provides an easy-to-use interface to work with various speech recognition APIs. It allows you to convert spoken language into text and supports multiple speech recognition engines, such as Google Web Speech API, Microsoft Bing Voice Recognition, and more.

Installation: To install the speech_recognition library, you can use pip:

pip install SpeechRecognition

Setting up with InfluxDB:

import influxdb_client
from influxdb_client import InfluxDBClient, Point, WritePrecision
from influxdb_client.client.write_api import SYNCHRONOUS

- influxdb_client:

  - This library allows you to interact with InfluxDB, a time series database, using Python. It provides functionalities to read and write data to the database, perform queries, and manage database operations.

- InfluxDBClient, Point, WritePrecision:

  - InfluxDBClient is a class that provides the necessary methods to establish a connection to an InfluxDB server. It allows you to configure connection settings such as the server's URL and authentication credentials.

  - Point represents a data point in InfluxDB. It lets you define measurements, tags, fields, and a timestamp for the data you want to write.

  - WritePrecision is an enumeration that specifies the precision of the timestamp when writing data points to InfluxDB.

Setting up with Datetime and timedelta:

from datetime import timedelta, datetime

- datetime is a module that supplies classes for manipulating dates and times. It's used to work with timestamps and time-related calculations.

- timedelta is a class within the datetime module that represents the difference between two dates or times. It's useful for adding or subtracting time intervals.

Setting up with TTS:

import pyttsx3

This library is a text-to-speech (TTS) engine that allows you to convert text into speech. It's useful for applications where you want your computer or device to speak out information to the user.

Installation: To install the pyttsx3 library, you can use pip:

pip install pyttsx3

Setting up with Telegram:

import telebot

Telebot is a Python library that interacts with the Telegram Bot API. It simplifies the process of creating and managing Telegram bots. You can use it to send and receive messages, multimedia, and other content through Telegram bot interactions.

CIRCUIT DIAGRAM:

The DHT11 (Humidity – temperature Sensor) has 3 pins GND , VCC , Signal

- VCC is connected with 3.3v

- GND is connected with Gnd

- Signal is connected with D15

The SCTO13 that whole connection as we can see has 3 pins GND , Output , VCC

- VCC is connected with 5v

- GND is connected with same Gnd

- Output pin is connected with D14

CONNECTION DIAGRAM :

SAMPLE UTTERANCES:

I have tested the below given utterances for opening and closing and it was working fine. More phrases can be possible:

Recommendation:

- System What is the right time to turn on the AC today

- System What is the best time to turn on the AC today

- System What is the recommended time to turn on the Ac

- System Suggest a Suitable table to turn on the AC

- System Recommend best time to turn on the AC today

Output:

Its giving voice responses and also giving responses in telegram

Output :

Video :

Code Explanation:

Python source code :

Sending values to influxdb :

Code:

import requests
import time
import influxdb_client
import paho.mqtt.client as mqtt
from influxdb_client import InfluxDBClient, Point, WritePrecision
from influxdb_client.client.write_api import SYNCHRONOUS


class InfluxDB:
    def __init__(self, bucket="Weather_Test_Data"):
        self.token = "4S-CyOmTPUhSgUiUSf0A8Fn31keTgErGXCoxoN1makt745rOxeASQAczKo_9K8IK7i-BZDq6L2bg7Y7Qr20qLQ=="
        self.org = "Christ"
        # self.url = "http://localhost:8086"
        self.url = "http://10.21.70.16:8086"
        self.bucket = bucket
        self.client = influxdb_client.InfluxDBClient(url=self.url, token=self.token, org=self.org)
        self.write_api = None
        self.delete_api = None
        self.query_api = None

    def delete(self, measure, start, stop):
        if self.delete_api is None:
            self.delete_api = self.client.delete_api()

        #start = "1970-01-01T00:00:00Z"
        #stop = "2100-01-01T00:00:00Z"
        return self.delete_api.delete(start, stop, "_measurement="+measure, bucket=self.bucket, org=self.org)

    def write(self, measure, atag, afield):
        if self.write_api is None:
            self.write_api = self.client.write_api(write_options=SYNCHRONOUS)

        point = (Point(measure).tag(atag[0], atag[1]).field(afield[0], afield[1]))
        return self.write_api.write(bucket=self.bucket, org=self.org, record=point)

    def query(self, measure, start):
        if self.query_api is None:
            self.query_api = self.client.query_api()

        query = f"""from(bucket: "{self.bucket}")
         |> range(start: {start})
         |> filter(fn: (r) => r._measurement == "{measure}")"""
        tables = self.query_api.query(query, org=self.org)

        return tables

    def query_mean(self, measure, start):
        if self.query_api is None:
            self.query_api = self.client.query_api()

        query = f"""from(bucket: "{self.bucket}")
         |> range(start: {start})
         |> filter(fn: (r) => r._measurement == "{measure}")
         |> mean()"""
        tables = self.query_api.query(query, org=self.org)

        return tables

# Your existing InfluxDB class definition

# Your existing code for fetching outdoor weather data
def fetch_weather_data(city_name):
    api_key = '9e92654b6666b69de295338624d1ac4e'
    base_url = f'https://api.openweathermap.org/data/2.5/weather?q={city_name}&appid={api_key}&units=metric'

    try:
        response = requests.get(base_url)
        data = response.json()

        if response.status_code == 200:
            temperature = data['main']['temp']
            humidity = data['main']['humidity']
            return temperature, humidity
        else:
            print('Error:', data['message'])
            return None, None

    except Exception as e:
        print('Error:', str(e))
        return None, None


# Your existing code for measuring indoor data


# MQTT on_message callback function
# ... (previous code)

def on_message(client, userdata, message):
    if message.topic == "tempTopic":
        temperature = float(message.payload.decode("utf-8"))
        print("Received Temperature:", temperature)
        ts.write("testing", ["location", "indoor"], ["temperature", temperature])  # Indoor temperature
    elif message.topic == "humiTopic":
        humidity = float(message.payload.decode("utf-8"))
        print("Received Humidity:", humidity)
        ts.write("testing", ["location", "indoor"], ["humidity", humidity])  # Indoor humidity

        # Fetch outdoor weather data
        city_name = 'Kolkata'
        outdoor_temperature, outdoor_humidity = fetch_weather_data(city_name)

        if outdoor_temperature is not None and outdoor_humidity is not None:
            print(f'Outdoor Temperature in {city_name}: {outdoor_temperature}°C')
            print(f'Outdoor Humidity in {city_name}: {outdoor_humidity}%')
            ts.write("outdoor_temperature", ["location", "outdoor"], ["temperature", outdoor_temperature])
            ts.write("outdoor_humidity", ["location", "outdoor"], ["humidity", outdoor_humidity])

    elif message.topic == "AcStatusTopic":
        Ac_Status = int(message.payload.decode("utf-8"))
        print("Received AC_Status:", Ac_Status)
        ts.write("testing", ["location", "indoor"], ["Ac_Status", Ac_Status])





if __name__ == '__main__':

    measure = "testing"  # Specify your measurement name

    ts = InfluxDB(bucket="Weather_Test_Data")

    # Set up MQTT client and connect to the broker using the broker address
    mqtt_client = mqtt.Client()
    mqtt_client.on_message = on_message
    mqtt_client.connect("10.21.70.16", 1883)  # Replace with your MQTT broker address
    mqtt_client.subscribe([("tempTopic", 0), ("humiTopic", 0),("AcStatusTopic" , 0)])
    mqtt_client.loop_start()

    try:
        while True:
            time.sleep(120)  # Sleep for 2 minutes
    except KeyboardInterrupt:
        mqtt_client.disconnect()
        mqtt_client.loop_stop()

Explanation:

1. **InfluxDB Class:**

- `__init__(self, bucket="Weather_Data")`: Initializes the InfluxDB class with default or user-provided values for token, organization (org), URL, and bucket.

- `delete(self, measure, start, stop)`: Deletes data points within the specified time range and measurement from the InfluxDB bucket.

- `write(self, measure, atag, afield)`: Writes data points with the specified measurement, tags, and fields to the InfluxDB bucket.

- `query(self, measure, start)`: Performs a Flux query to retrieve data points of the specified measurement from the InfluxDB bucket within the given time range.

- `query_mean(self, measure, start)`: Similar to `query`, but calculates the mean value of data points within the specified measurement and time range.

2. **fetch_weather_data Function:**

- `fetch_weather_data(city_name)`: Fetches outdoor weather data from OpenWeatherMap API for the provided `city_name`.

- Constructs the API URL, sends a GET request, and parses the response JSON.

- Returns outdoor temperature and humidity if the API request is successful; otherwise, returns `None, None`.

3. **on_message Callback Function:**

- `on_message(client, userdata, message)`: Callback function triggered when an MQTT message is received.

- Handles messages for "tempTopic," "humiTopic," and "AcStatusTopic."

- Decodes message payloads, updates InfluxDB with indoor temperature, humidity, and AC status data, and fetches and writes outdoor weather data.

- Handles exceptions and error cases during data retrieval and writing.

4. **Main Execution Block:**

- Initializes the `InfluxDB` class instance `ts` with the specified bucket name ("Weather_Data").

- Sets up the MQTT client (`mqtt_client`) and connects it to the specified MQTT broker address and port.

- Subscribes to MQTT topics ("tempTopic," "humiTopic," and "AcStatusTopic") with QoS level 0.

- Starts the MQTT client's loop to listen for incoming MQTT messages.

5. **Main Loop:**

- The script enters an infinite loop (`while True`) for continuous execution.

- It periodically sleeps for 2 minutes (`time.sleep(120)`) to manage resource usage.

- The loop continues until a KeyboardInterrupt (`Ctrl+C`) is detected.

6. **Exception Handling:**

- Inside the `except KeyboardInterrupt` block, the script disconnects the MQTT client (`mqtt_client.disconnect()`) and stops its loop (`mqtt_client.loop_stop()`).

Overall, the code establishes a connection to an MQTT broker, subscribes to MQTT topics, processes received messages to update the InfluxDB database with temperature, humidity, and AC status data, fetches outdoor weather data, and maintains a loop for continuous execution until manually interrupted.

Fetching the data from influxdb for ChatGPT recommendation:

Code :

import influxdb_client
from influxdb_client import InfluxDBClient, Point, WritePrecision
from influxdb_client.client.write_api import SYNCHRONOUS
from datetime import timedelta, datetime
import openai
import re
import speech_recognition as sr
import pyttsx3
import telebot


class InfluxDB:
    def __init__(self, bucket="Weather_Test_Data"):
        self.token = "4S-CyOmTPUhSgUiUSf0A8Fn31keTgErGXCoxoN1makt745rOxeASQAczKo_9K8IK7i-BZDq6L2bg7Y7Qr20qLQ=="
        self.org = "Christ"
        # self.url = "http://localhost:8086"
        self.url = "http://10.21.70.16:8086"
        self.bucket = bucket
        self.client = influxdb_client.InfluxDBClient(url=self.url, token=self.token, org=self.org)
        self.write_api = None
        self.delete_api = None
        self.query_api = None

    def delete(self, measure, start, stop):
        if self.delete_api is None:
            self.delete_api = self.client.delete_api()

        #start = "1970-01-01T00:00:00Z"
        #stop = "2100-01-01T00:00:00Z"
        return self.delete_api.delete(start, stop, "_measurement="+measure, bucket=self.bucket, org=self.org)

    def write(self, measure, atag, afield):
        if self.write_api is None:
            self.write_api = self.client.write_api(write_options=SYNCHRONOUS)

        point = (Point(measure).tag(atag[0], atag[1]).field(afield[0], afield[1]))
        return self.write_api.write(bucket=self.bucket, org=self.org, record=point)

    # def query(self, measure, start):
    #     if self.query_api is None:
    #         self.query_api = self.client.query_api()
    #
    #     start = "2023-07-29T00:00:00Z"
    #
    #     query = f"""from(bucket: "{self.bucket}")
    #      |> range(start: {start})
    #      |> filter(fn: (r) => r._measurement == "{measure}")"""
    #     tables = self.query_api.query(query, org=self.org)
    #
    #     return tables

    def query_mean(self, measure, start):
        if self.query_api is None:
            self.query_api = self.client.query_api()

        query = f"""from(bucket: "{self.bucket}")
         |> range(start: {start})
         |> filter(fn: (r) => r._measurement == "{measure}")
         |> mean()"""
        tables = self.query_api.query(query, org=self.org)

        return tables


    def get_indoor_humidity(self, start_date, end_date):
        query = f"""from(bucket: "{self.bucket}")
               |> range(start: {start_date.strftime('%Y-%m-%dT%H:%M:%SZ')}, stop: {end_date.strftime('%Y-%m-%dT%H:%M:%SZ')})
               |> filter(fn: (r) => r._measurement == "indoor_humidity" and r._field == "value" and r.location == "indoor")
               |> window(every: 1d)
               |> aggregateWindow(every: 1h, fn: mean, createEmpty: false)"""

        result = self.query(query)

        humidity_data_by_day = {day: [] for day in range(1, 4)}

        for table in result:
            for record in table.records:
                timestamp = record["_time"]
                day = (timestamp.date() - start_date.date()).days + 1
                humidity = record["_value"]
                humidity_data_by_day[day].append(humidity)

        return humidity_data_by_day

    def get_outdoor_humidity(self, start_date, end_date):
        query = f"""from(bucket: "{self.bucket}")
               |> range(start: {start_date.strftime('%Y-%m-%dT%H:%M:%SZ')}, stop: {end_date.strftime('%Y-%m-%dT%H:%M:%SZ')})
               |> filter(fn: (r) => r._measurement == "testing" and r._field == "humidity" and r.location == "outdoor")
               |> window(every: 1d)
               |> aggregateWindow(every: 1h, fn: mean, createEmpty: false) """

        result = self.query(query)

        humidity_data_by_day = {day: [] for day in range(1, 4)}

        for table in result:
            for record in table.records:
                try:
                    timestamp = record["_time"]
                    day = (timestamp.date() - start_date.date()).days + 1
                    humidity = record["_value"]
                    humidity_data_by_day[day].append(humidity)
                except:
                    continue

        return humidity_data_by_day

    def get_outdoor_temperature(self, start_date, end_date):
        query = f"""from(bucket: "{self.bucket}")
               |> range(start: {start_date.strftime('%Y-%m-%dT%H:%M:%SZ')}, stop: {end_date.strftime('%Y-%m-%dT%H:%M:%SZ')})
               |> filter(fn: (r) => r._measurement == "testing" and r._field == "temperature" and r.location == "outdoor")
               |> window(every: 1d)
               |> aggregateWindow(every: 1h, fn: mean, createEmpty: false)"""

        result = self.query(query)

        temperature_data_by_day = {day: [] for day in range(1, 4)}

        for table in result:
            for record in table.records:
                try:
                    timestamp = record["_time"]
                    day = (timestamp.date() - start_date.date()).days + 1
                    temperature = record["_value"]
                    temperature_data_by_day[day].append(temperature)
                except:
                    continue
        return temperature_data_by_day

    def get_indoor_temperature(self, start_date, end_date):
        query = f"""from(bucket: "{self.bucket}")
            |> range(start: {start_date.strftime('%Y-%m-%dT%H:%M:%SZ')}, stop: {end_date.strftime('%Y-%m-%dT%H:%M:%SZ')})
            |> filter(fn: (r) => r._measurement == "testing" and r._field == "temperature" and r.location == "indoor")
            |> window(every: 1d)
            |> aggregateWindow(every: 1h, fn: mean, createEmpty: false)"""

        result = self.query(query)

        temperature_data_by_day = {day: [] for day in range(1, 4)}

        for table in result:
            for record in table.records:
                try:
                    timestamp = record["_time"]
                    day = (timestamp.date() - start_date.date()).days + 1
                    temperature = record["_value"]
                    temperature_data_by_day[day].append(temperature)
                except:
                    continue

        return temperature_data_by_day

    def get_ac_status(self, start_date, end_date):
        query = f"""from(bucket: "{self.bucket}")
            |> range(start: {start_date.strftime('%Y-%m-%dT%H:%M:%SZ')}, stop: {end_date.strftime('%Y-%m-%dT%H:%M:%SZ')})
            |> filter(fn: (r) => r._measurement == "testing" and r._field == "AC_Status" and r.location == "indoor")
            |> window(every: 1d)
            |> aggregateWindow(every: 1h, fn: mean, createEmpty: false)"""

        result = self.query(query)

        ac_status_data_by_day = {day: [] for day in range(1, 4)}

        for table in result:
            for record in table.records:
                try:
                    timestamp = record["_time"]
                    day = (timestamp.date() - start_date.date()).days + 1
                    ac_status = list(map(round,record["_value"]))
                    ac_status_data_by_day[day].append(ac_status)
                except:
                    continue

        return ac_status_data_by_day

    def get_time_data(self):


        time_data_by_day = ['00:00:00', '01:00:00', '02:00:00', '03:00:00', '04:00:00', '05:00:00', '06:00:00', '07:00:00', '08:00:00', '09:00:00', '10:00:00', '11:00:00', '12:00:00', '13:00:00', '14:00:00', '15:00:00', '16:00:00', '17:00:00', '18:00:00', '19:00:00', '20:00:00', '21:00:00', '22:00:00', '23:00:00']


        return time_data_by_day



    def query(self, query):
        if self.query_api is None:
            self.query_api = self.client.query_api()

        return self.query_api.query(query, org=self.org)

    def get_formatted_data(self, measurement, field, location, start_date, end_date):
        query = f"""from(bucket: "{self.bucket}")
            |> range(start: {start_date.strftime('%Y-%m-%dT%H:%M:%SZ')}, stop: {end_date.strftime('%Y-%m-%dT%H:%M:%SZ')})
            |> filter(fn: (r) => r._measurement == "{measurement}" and r._field == "{field}" and r.location == "{location}")
            """

        result = self.query(query)

        formatted_data_by_day = {}

        for table in result:
            for record in table.records:
                timestamp = record["_time"]
                day = (timestamp.date() - start_date.date()).days + 1

                if day not in formatted_data_by_day:
                    formatted_data_by_day[day] = {
                        "time": [],
                        "indoor Humidity": [],
                        "Outdoor Humidity Data": [],
                        "Indoor Temperature Data": [],
                        "Outdoor Temperature Data": [],
                        "AC_Status Data": []
                    }

                formatted_data_by_day[day]["time"].append(timestamp.strftime("%H:%M:%S"))
                formatted_data_by_day[day]["indoor Humidity"].append(record["_value"])
                
        return formatted_data_by_day


token = "4S-CyOmTPUhSgUiUSf0A8Fn31keTgErGXCoxoN1makt745rOxeASQAczKo_9K8IK7i-BZDq6L2bg7Y7Qr20qLQ=="  # Replace with your InfluxDB token  # Replace with your InfluxDB token
bucket = "Weather_Test_Data"

# Calculate the start date (last three days from July 31, 2023)
end_date = datetime(2023, 8, 1)
start_date = end_date - timedelta(days=3)

# Create an InfluxDB instance without passing the 'token' argument
influx_db = InfluxDB()

# Fetch indoor humidity data using the class method
indoor_humidity_data = influx_db.get_indoor_humidity(start_date, end_date)

# Fetch outdoor humidity data using the class method
outdoor_humidity_data = influx_db.get_outdoor_humidity(start_date, end_date)

# Fetch outdoor temperature data using the class method
outdoor_temperature_data = influx_db.get_outdoor_temperature(start_date, end_date)

# Fetch indoor temperature data using the class method
indoor_temperature_data = influx_db.get_indoor_temperature(start_date, end_date)

# Fetch AC_Status data using the class method
ac_status_data = influx_db.get_ac_status(start_date, end_date)

# Fetch time data using the class method
time_data = influx_db.get_time_data()


# Fetch and format data using the class method
formatted_data = influx_db.get_formatted_data("testing", "humidity", "indoor", start_date, end_date)

# Add outdoor humidity data
formatted_data_outdoor_humidity = influx_db.get_formatted_data("testing", "humidity", "outdoor", start_date, end_date)
for day, data in formatted_data_outdoor_humidity.items():
    formatted_data[day]["Outdoor Humidity Data"] = data["indoor Humidity"]

# Add indoor temperature data
formatted_data_indoor_temp = influx_db.get_formatted_data("testing", "temperature", "indoor", start_date, end_date)
for day, data in formatted_data_indoor_temp.items():
    formatted_data[day]["Indoor Temperature Data"] = data["indoor Humidity"]

# Add outdoor temperature data
formatted_data_outdoor_temp = influx_db.get_formatted_data("testing", "temperature", "outdoor", start_date, end_date)
for day, data in formatted_data_outdoor_temp.items():
    formatted_data[day]["Outdoor Temperature Data"] = data["indoor Humidity"]

# Add AC status data
formatted_data_ac_status = influx_db.get_formatted_data("testing", "AC_Status", "indoor", start_date, end_date)
for day, data in formatted_data_ac_status.items():
    formatted_data[day]["AC_Status Data"] = data["indoor Humidity"]

# Print the formatted data for each day
for day, data in formatted_data.items():
    print(f"Day {day} Data:")
    print(data)
    print()

# ... (previous code)

# Create a dictionary to store the combined data for each day
combined_data = {}


# Combine all the data into the desired format
for day, data in formatted_data.items():
    combined_data[day] = {
        "time": data["time"],
        "indoor Humidity Data": data["indoor Humidity"],
        "Outdoor Humidity Data": formatted_data_outdoor_humidity[day]["indoor Humidity"],
        "Indoor Temperature Data": formatted_data_indoor_temp[day]["indoor Humidity"],
        "Outdoor Temperature Data": formatted_data_outdoor_temp[day]["indoor Humidity"],
        "AC_Status Data": formatted_data_ac_status[day]["indoor Humidity"]
    }

# Print the combined data dictionary
print(combined_data)

# Set your OpenAI API key here
openai.api_key = "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"

# Define the data
data = combined_data


# Function to recognize speech using the microphone
def recognize_speech():
    recognizer = sr.Recognizer()

    with sr.Microphone() as source:
        print("Say something:")
        audio = recognizer.listen(source)

    try:
        recognized_text = recognizer.recognize_google(audio)
        print("You said:", recognized_text)
        return recognized_text
    except sr.UnknownValueError:
        print("Speech Recognition could not understand the audio.")
    except sr.RequestError as e:
        print(f"Could not request results from Google Web Speech Recognition service; {e}")
    except Exception as e:
        print("Unknown error:", e)
        return None


# Function to analyze data using OpenAI
def analyze_data(question):
    try:
        # Check if the recognized text starts with "System"
        if question.lower().startswith("system"):
            # prompt = f"""System: Analyse the following last three days time series  data  which is json format and tell me the time period based on the question. Give answer like as recommendation
            # - the (suitable or perfect or right or recommend ) time to turn on the AC (from when (AM or PM )to when(AM or PM) ) and give recomendation  why AC should be turn on in creative way
            # - do not give strange time interval like at night timing from 10 PM to 5AM dont recomend this time period
            #
            # Data: {data}
            #
            # User: {question}
            # """

            prompt =f""" Analyse the following last three days data which is in json format and recommend a time period when the ac should be turn on in morning and afternoo
                - give time period like (from when (AM or PM )to when(AM or PM) ) for each morning and afternoon
                - give reasons also
             Data: {data}
             
             User: {question}
            """

            response = openai.Completion.create(
                engine="text-davinci-003",
                prompt=prompt,
                max_tokens=150,
                temperature=0.7,
            )

            answer = response.choices[0].text.strip()

            time_match = re.search(r'\d{1,2}:\d{2} [APap][Mm]', answer)
            if time_match:
                extracted_time = time_match.group()
                answer = answer.replace("[time]", extracted_time)

            return answer
        else:
            return "Please start your question with 'System'."

    except Exception as e:
        return f"Error: {e}"


# Function to play TTS audio
# Function to convert text to speech using pyttsx3
def convert_to_speech(text):
    engine = pyttsx3.init()
    # Skip speaking the "Answer" part
    if text.lower().startswith("answer"):
        text = text.split(" ", 1)[1]
    elif text.lower().startswith("system"):
        text = text.split(" ", 1)[1]
    engine.say(text)
    engine.runAndWait()



bot_token = '6505007326:AAFsm0gFmkccJR8zO3LAaoO0zwKUW_Fv2BE'
bot = telebot.TeleBot(bot_token)

# Function to send a message
def send_telegram_message(message):
    # Replace with the actual chat ID where you want to send the message
    bot_chat_id = '5906755016'


    # Check the message before sending
    if not message.strip().lower().startswith("please start your question with 'system'"):
        # Send the message
        bot.send_message(chat_id=bot_chat_id, text=message)



# Main program
def main():
    while True:
        question = recognize_speech()
        if question:
            answer = analyze_data(question)
            print(f"Q: {question}\nA: {answer}")
            convert_to_speech(answer)
            if answer.lower().startswith("answer:"):
                answer = answer.split(" ", 1)[1]
            # Send the message using the function
                send_telegram_message(answer)


if __name__ == "__main__":
    main()

Explanation:

- InfluxDB Class (Partial):

  - __init__(self, bucket="Weather_Data"): Constructor initializes the InfluxDB client with the provided token, organization, URL, and bucket.

  - delete(self, measure, start, stop): Deletes data points from the specified measurement within the given time range.

  - write(self, measure, atag, afield): Writes data points to the specified measurement with provided tags and fields.

  - query_mean(self, measure, start): Executes a mean aggregation query on the specified measurement within the given time range.

- Additional InfluxDB Methods:

  - get_indoor_humidity, get_outdoor_humidity, get_outdoor_temperature, get_indoor_temperature, get_ac_status, get_time_data, get_formatted_data: These methods retrieve specific data from the InfluxDB database within a specified time range and organize it by days.

- Data Collection:

  - Fetches indoor and outdoor humidity, indoor and outdoor temperature, and AC status data using the InfluxDB class methods.

  - Retrieves data for the last three days.

- OpenAI and Analysis Functions:

  - analyze_data(question): Utilizes OpenAI's GPT-3 model to analyze the user's question and generate a response.

  - Creates a prompt incorporating the question and data, asking for a suitable time to turn on the AC with reasons.

- Voice Recognition and TTS:

  - recognize_speech(): Uses the microphone to capture user speech, converts it to text using Google Web Speech Recognition.

  - convert_to_speech(text): Converts text to speech using the pyttsx3 library for audio output.

- Telegram Integration:

  - send_telegram_message(message): Sends a message to a specified Telegram chat using the telebot library.

- Main Program:

  - Initializes a loop that continuously listens for user speech input.

  - If user input is detected, it is recognized, analyzed using OpenAI, and the response is converted to speech.

  - The response is also sent to the specified Telegram chat using the send_telegram_message function.

Arduino Source code :

Code:

#include <Ethernet.h>

#include <Wire.h>

#include <SoftWire.h>

#include <DHT.h>

#include <PubSubClient.h>

#include <ArduinoJson.h>

#include "HardwareSerial.h"

// DHT sensor setup

#define DHTPIN 15     // Pin where the DHT11 sensor is connected

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

// AC current setup

#define ACPIN 14

#define ACON_THRESHOLD 200

// Update these with values suitable for your network.

byte mac[]    = {  0xDE, 0xED, 0xBA, 78, 0xFE, 0xED };

IPAddress ip(172, 16, 0, 100);

//IPAddress server(44, 195, 202, 69);

IPAddress server(10, 21, 70, 16);

void callback(char* topic, byte* payload, unsigned int length) {

// Handle incoming MQTT messages based on the topic if needed

}

EthernetClient ethClient;

PubSubClient client(ethClient);

void reconnect() {

// Loop until we're reconnected

while (!client.connected()) {

Serial.print("Attempting MQTT connection...");

// Attempt to connect

if (client.connect("arduinoClient78")) {

Serial.println("connected");

// Once connected, resubscribe to the topics

client.subscribe("tempTopic");

client.subscribe("humiTopic");

client.subscribe("ACStatusTopic");

} else {

Serial.print("failed, rc=");

Serial.print(client.state());

Serial.println(" try again in 5 seconds");

// Wait 5 seconds before retrying

delay(5000);

}

}

}

void setup() {

// Open serial communications and wait for port to open:

Serial3.setRx(PC11);

Serial3.setTx(PC10);

delay(50);

Serial.begin(115200);

//Ethernet.init(17);

dht.begin();

client.setServer(server, 1883);

client.setCallback(callback);

Ethernet.begin(mac);

// Allow the hardware to sort itself out

delay(1500);

}

void loop() {

if (!client.connected()) {

reconnect();

}

client.loop();

// Read temperature and humidity from the sensor

float humidity = dht.readHumidity();

float temperature = dht.readTemperature();

if (isnan(humidity) || isnan(temperature)) {

Serial.println("Failed to read from DHT sensor!");

return;

}

Serial.print("Temperature: ");

Serial.print(temperature);

Serial.print(" °C\t");

Serial.print("Humidity: ");

Serial.print(humidity);

Serial.println(" %");

// Convert readings to strings

String tempStr = String(temperature);

String humStr = String(humidity);

// Publish temperature and humidity readings to MQTT topics

client.publish("tempTopic", tempStr.c_str());

client.publish("humiTopic", humStr.c_str());

// Publish AC status to MQTT topics

int combinedCurrent = analogRead(ACPIN);

Serial.println(combinedCurrent);

if (combinedCurrent > ACON_THRESHOLD) {

Serial.println("AC Status: ON");

client.publish("ACStatusTopic", "1");

} else {

Serial.println("AC Status: OFF");

client.publish("ACStatusTopic", "0");

}

// Wait for 2 minue before taking the next reading

//delay(120000);

delay(6000);

}

Explanation:

- Callback Function (callback):

  - This function is intended to handle incoming MQTT messages based on the subscribed topics.

  - However, the actual handling logic is not implemented in the code snippet provided.

- Reconnection Function (reconnect):

  - This function is responsible for attempting to reconnect to the MQTT broker if the client is not currently connected.

  - It goes into a loop until the client successfully reconnects.

  - If reconnection fails, it prints the client's state and waits for 5 seconds before trying again.

- Setup Function (setup):

  - Initializes the hardware and configurations during the setup phase of the Arduino sketch.

  - Configures serial communication with the Serial3 interface.

  - Begins serial communication with a baud rate of 115200.

  - Initializes the DHT sensor to read data from the specified pin and of the specified type (DHT11).

  - Sets up the PubSubClient with the MQTT server and callback function.

  - Initializes the Ethernet communication and allows time for the Ethernet hardware to initialize (1500 ms).

- Main Loop Function (loop):

  - The main loop of the Arduino sketch, which continuously executes after the setup phase.

  - Checks if the MQTT client is not connected, and if so, attempts to reconnect using the reconnect function.

  - Executes the PubSubClient's loop function, which handles MQTT communication.

  - Reads the temperature and humidity data from the DHT sensor.

  - Checks if the sensor readings are valid (not NaN), and if they are, publishes the temperature and humidity readings to their respective MQTT topics ("tempTopic" and "humiTopic").

  - Reads the combined current from an analog pin (ACPIN), which is presumably connected to an AC current sensor.

  - Compares the combined current value with a predefined threshold (ACON_THRESHOLD) to determine whether the AC is ON or OFF.

  - Prints the AC status and publishes it to the "ACStatusTopic" MQTT topic as "1" (ON) or "0" (OFF).

  - Adds a delay of 6 seconds before the next iteration of the loop.

Overall, this code combines reading data from a DHT11 sensor, monitoring AC current, and publishing these readings to MQTT topics using an Ethernet connection.